SCOPE OF WORK

Daybreak Sego 3 Construction Staking

Survey Services

- Construction Staking

Provide the following construction survey staking services:

- Sanitary Sewer – Stake sewer laterals according to the existing lot plan and existing utilities from KLC base files.

- Water Line – Stake water laterals for installation according the lot plan and existing utilities from KLC base files.

- Dry Utilities – Stake electric and gas boxes

Note:  Staking is intended for one (1) time only and does not include extra staking due to stakes being removed, destroyed or lost. Additional staking will be at the expense and written approval of the contractor.

- Monumentation

We will set copper rivet witness corners in top back of curb (TBC) at each of the front lot lines.  Set rebar, cap and tee post at each rear lot corner.  On alley loaded lots copper rivets will be set in the alley concrete.

- Sewer Certification (if necessary)

Field locate the sewer manhole after it has been constructed if one is required and prepare a letter to certify where they are located horizontally and vertically within the plat.

- Topographic Survey

Locate existing improvements and utilities at tie in points. Prepare AutoCad drawing with above described items and surface.

- Building Staking

Stake corners of building for dig and footing for each of the 6 buildings on the site. Assumes 2 mobilizations for each building.

- Driveway & Sidewalk Staking

Stake driveway tie-in locations for the main driveway access as well as the island as shown in the site plan. Stake sidewalk in the open space areas according to the site plan.